上機考期末

指令：Mechanical APDL Command Reference (bme.hu)

FINISH

/CLEAR

/prep7

PCIRC, 10, 20, -30, 30  (創建以工作平面原點為中心之圓形區域：PCIRC,圓內徑,圓外徑,起始角度,結束角度)

RECTNG, 0, 70, -2.5, 2.5 (在工作平面上的任意位置建立矩形區域：RECTNG, x1, x2, y1, y2)

AOVLAP, ALL   (重疊區選取)

ADELE, 6, , , 1  (刪除區域：ADELE, 第幾個區域, , , 1為刪除區域(0)和關鍵點)

ET, 100, 200, 6      ! 3D QUAD 4-NODE since this corresponds to an element face of SOLID45

ET, 1, 45           *定義種類(ET,元素參考號碼,元素編號)

MP, EX, 1, 200000   *定義線性材料性質(MP,性質代號,材料性質編號,E)

MP, DENS, 1, 7.8E-9    *lab：EX(楊氏模數 E), PRXY/NUXY(蒲松比 v),密度DENS

MP, NUXY, 1, 0.3

ESIZE, 4        *設定元素大小

TYPE, 100  *設置元素類型屬性指標

MSHKEY, 1 * 1 使用映射網格劃分網格。

AMESH, ALL  *面上生成節點單元

TYPE, 1

MAT, 1

ESIZE,   ,5                   ! NDIV

VEXT,ALL, , ,0,0,-50    *沿坐標軸方向延伸距離並建立體積(ALL:NA2、NINC可審略),各軸延伸距離

CSYS, 1      *啟動先前定義的座標系(查表),1為圓柱形Z 作為旋轉軸

VGEN,6,ALL, , ,0,60,0  *生成六個葉片,素材,,,座標

NUMMRG, NODE     *合併重複的節點

NUMMRG, KP        *合併重複的點

FINISH

/SOLU

CSYS, 1

NSEL, S, LOC, X, 10    *選擇節點的子集

D, ALL, ALL, 0    *定義節點處的自由度約束

! CSYS, 0

OMEGA,0,0,209.44      *指定結構的旋轉速度(OMEGX、OMEGY、OMEGZ)

ALLSEL              *使用單個命令選擇所有實體

SOLVE

FINISH

/POST1

PLNSOL,S,EQV

FINISH

/CLEAR

!/單位mm,N,MPa

/PREP7

ET,1,PLANE42

KEYOPT,1,3,1         ! 軸對稱模型

MP,EX,1,210000       !  E=210 GPa

MP,NUXY,1,0.3        !  ν=0.3

BLC4,200,0,300,100

ESIZE,10

TYPE,1

MAT,1

MSHKEY,1

AMESH,1

EPLOT

FINISH

/SOLU

ANTYPE,STATIC

DL,1,,SYMM    !將ex5-5平面的邊界條件換成軸對稱角度思考

DL,3,,SYMM

DL,4,,ALL,0

SFL,2,PRES,1,

SOLVE

FINISH

/POST1

PLNSOL,S,X,0,1

PLNSOL,S,Z,0,1

PATH,stress,2,30,60,

PPATH,1,0,200,0,0,0,

PPATH,2,0,500,0,0,0,

PDEF, ,S,X,AVG

PDEF, ,S,Z,AVG

PLPATH,SX,SZ

PRPATH,SX,SZ

FINISH

/CLEAR

B=0.003

H=0.006

AREA=B*H

IZ=(1/12)*B*H*H*H

/PREP7

ET,1,BEAM3     *element type BEAM3

R,1, AREA, IZ ,H , ,  *特性參數 ( R,參考編號,參數(R1-R6)) 面積、力矩慣性、厚度

MP,EX,1,2200e6

MP,DENS,1,1100

K,1,0,0,,         *關鍵點建立 K,點編號,X 值,Y 值

K,2,0.3,0,,

LSTR,1,2         *將關鍵點連線

TYPE,1

MAT,1

REAL,1

LESIZE,1, , ,70, , , , ,1   將線段劃分(對所選擇線設置網格單元大小)

LESIZE,選擇線段數, , ,number of divisions per line, , , , ,YES(1)

LMESH,1             *在線上生成節點單元

EPLOT                *生成元素顯示

DK,1,ALL,0, ,0,         *固定端: 點 1 所有自由度固定

FINISH

/SOLU

ANTYPE,MODAL

MODOPT,LANB,10,0,500,    *設定模態抓取  MODOPT,LANB(block lanczos method),模態抓取數目,0 至 500HZ

MXPAND,10               *模態擴展數設定 10 個

SOLVE

FINISH

/POST1

SET,LIST

SET,1,1

PLDISP,1

FINISH

/CLEAR

B=0.003

H=0.006

AREA=B*H

IZ=(1/12)*B*H**3

/PREP7

ET,1,BEAM3

R,1, AREA, IZ ,H , ,

MP,EX,1,2200e6

MP,DENS,1,1100

K,1,0,0,,

K,2,0.3,0,,

LSTR,1,2

TYPE,1

MAT,1

REAL,1

LESIZE,1, , ,70, , , , ,1

LMESH, 1

EPLOT

DK,1,UX

DK,1,UY

DK,1,UZ

DK,2,UY

FINISH

/SOLU

ANTYPE,MODAL

MODOPT,LANB,10,0,700,

MXPAND,10

SOLVE

FINISH

/POST1

SET,LIST

SET,1,1

PLDISP,1

FINISH

/CLEAR

B=0.003

H=0.006

AREA=B*H

IZ=(1/12)*B*H**3

/PREP7

ET,1,BEAM3

R,1, AREA, IZ ,H , ,

MP,EX,1,2200e6

MP,DENS,1,1100

K,1,0,0,,

K,2,0.3,0,,

LSTR,1,2

TYPE,1

MAT,1

REAL,1

LESIZE,1, , ,70, , , , ,1

LMESH, 1

EPLOT

DK,1,ALL,0, ,0,

FINISH

/SOLU

ANTYPE,HARMIC

HROPT,FULL        *設定求解法

HROUT,OFF          *計算結果檔呈現形式

HARFRQ, 0, 500     *計算範圍 (頻率)

NSUBST,100         *將上述範圍切成100等分計算

KBC,1                     *負荷方式設定為階梯式

OUTRES,ALL,ALL       *表示於結果檔案(* .rst)中，每個頻率結果均儲存下來。

DMPRAT, 0.01           *整個結構之阻尼比為0.01

F, 2,FY, 0.001,0

!FK,2,FY, 0.001,0

SOLVE

FINISH

/POST26

NSOL,2,2,U,Y,END-UY      節點2(懸臂樑之右端自由端)的UY位移抓出，定義為變數2,並取名

PRCPLX,1         設定分析結果輸出為振幅和相位角 (Amplitude+Phase)格式

PLCPLX,0         分析結果的繪圖為振幅值

PLVAR,2, , , , , , , , , ,  變數2(自由端的UY位移，單位為 m)為縱軸，以頻率值(單位為Hz)為橫軸，畫出位移振幅-頻率圖。

PRVAR,2,

/GROPT,LOGY,ON

PLVAR,2, , , , , , , , , ,   振幅log值-頻率圖

FINISH

/POST1

SET,LIST

SET,1,3

HRCPLX,1,3,-33.7943   「HRCPLX, 1, 3, -33.7943,」將相位角 為 -33.7943度的位移振幅算出

PLDISP,1

FINISH

/CLEAR

/PREP7

ET,1,BEAM3

R,1,0.4*0.2,1/12*0.2*0.064,0.4, , , ,   *特性參數 ( R,參考編號,參數(R1-R6)) 面積、力矩慣性、厚度

MP,EX,1,12E9

MP,DENS,1,1500

K,,0,0

K,,6,0

LSTR, 1, 2

TYPE,   1

MAT,       1

REAL,       1

ESIZE,0.2,0,

LMESH,1,

NPLOT                Displays nodes

EPLOT                Produces an element display

DK,1,ALL,0, ,0,

FINISH

/SOLU

ANTYPE,TRANS         (查表)

TRNOPT,FULL        指定瞬態分析選項(full method)

OUTRES,ALL,1       ! write every Nth substep (N=1)

ALPHAD,0              定義阻尼的品質矩陣乘法器

BETAD,0.006            定義阻尼的剛度矩陣乘數

TIME,0.0001           ! At time equals 0.0001s ≒ 0s

NSUBST,2          此負載要執行的子步驟數

TIMINT,OFF            開啟瞬態效果,無瞬態效應（靜態或穩態）

KBC,1                 ! Stepped load step

D, 2,UY,-0.04         ! Define the DOF constraints at the end "NODE"

SOLVE

TIME,2

DELTIM,0.01,0.005,0.02     ! Specifies the time step sizes

AUTOTS,ON                  ! Automatic time stepping

TIMINT,ON

DDELE, 2,UY           ! Delete the DOF constraints at the end "NODE"

SOLVE

FINISH

/POST26

NSOL,2,2,U,Y,END-UY       節點2(懸臂樑之右端自由端)的UY位移抓出，定義為變數2,並取名

PLVAR,2, , , , , , , , , ,       以圖形形式顯示最多十個變數

FINISH

/CLEAR

/FILNAME,ex8-1

/TITLE, ex8-1. Simple example for geometric nonlinearity

/PREP7

ET,1,PLANE42

KEYOPT,1,3,0      ! plane stress

!KEYOPT,1,3,3      ! plane stress with thickness

!R,1,0.005

MP,EX,1,210e9

MP,NUXY,1,0.3

K,,0,0,0

K,,1,0,0

K,,1,0.08,0

K,,0,0.08,0

A,1,2,3,4,

TYPE,1

MAT,1

ESIZE,0.02

AMESH,1

FINISH

/SOLU

ANTYPE,STATIC

NLGEOM,ON

AUTOTS,ON        ! automatic time stepping

NEQIT,200        ! number of equilibrium iterations

KBC,0                ! ramped loading

OUTRES,ALL,ALL   ! write all solution items for every substep

TIME,1

DELTIM,0.1,0.05,0.2      ! time step sizes

ALLSEL,ALL

DL,4, ,ALL,0

FK,2,FY,-1E7

SOLVE

FINISH

/POST1

/DSCALE,1,1.0     ! displacement scaling

PLNSOL,U,Y,1,1

PLNSOL,U,X,1,1

FINISH

/CLEAR

/TITLE,ex8-4. Elastoplastic analysis of a plate with a hole.

/PREP7

ET,1,PLANE42

KEYOPT,1,3,0       ! plane stress

MPTEMP,1,0           ! Young's modulus and Poisson's ratio at 0 degree Celsius (resets the previous value in STLOC to zero)

MPDATA,EX,1, ,71200

MPDATA,PRXY,1, ,0.31

TB,MISO,1,1,6               ! 6 data points to be specified for 1 given temperature

TBTEMP,0                    ! Temperature = 0 degree Celsius

TBPT,,0.0036517,260         ! 260/0.0036517=71200

TBPT,,0.0066,341.68

TBPT,,0.0083,358.34

TBPT,,0.0133,375.01

TBPT,,0.0266,408.34

TBPT,,0.0566,441.68

TBPLOT

K,1,0,0,0,

K,2,225,0,0,

K,3,0,450,0,

K,4,225,450,0,

KPLOT       *把點顯示出來

A,1,2,4,3         *面積畫出來

CYL4,0,0,112.5       *圓畫出來

ASBA,  1,  2    *面積1-2

APLOT

! SMRTSIZE,3

LESIZE,10, , ,15,0.2, , , ,1  *線分割(線號,,,畫分數,間距比,,,,0為關閉1為開啟)

LESIZE, 2, , ,20,3, , , ,1

LESIZE, 3, , ,5, , , , ,1

LESIZE, 9, , ,9, , , , ,1

LESIZE, 5, , ,13, , , , ,1

TYPE,1

MAT,1

MSHKEY,0      ! free mesh

AMESH,3

EPLOT

DL,10, ,UX,0    *定義線條上的自由度

DL, 9, ,UY,0

FINISH

/SOLU

ANTYPE,STATIC   *靜態分析

NLGEOM,ON     *要考慮偏轉

OUTRES,ALL,ALL   *控制寫入資料庫的解決方案數據

AUTOTS,ON        *指定是使用自動時間步進還是載入步進。

NEQIT,200

KBC,0             ! ramped

TIME,1

NSUBST,20,30,10

SFL,3,PRES, -163.33,

SOLVE

FINISH

/POST1

/DSCALE,1,1.0

SET,LAST

PLNSOL,S,EQV,0,1

PLNSOL,EPTO,Y,0,1

PLNSOL,EPPL,Y,0,1

PATH,center,2,30,60,

PPATH,1,0,112.5,0,0,0,

PPATH,2,0,  225,0,0,0,

PDEF,EYY,EPTO,Y,AVG     ! totoal strain of Y component

PLPATH,EYY

歷屆

FINISH

/CLEAR

W=10

L=20

T=1

AREA=W*T

I=(1/12)*T*W*W*W

/PREP7

ET,1,SHELL63

R,1, AREA, I ,W , ,

MP,EX,1,3e11

MP,DENS,1,7900

MP,NUXY,1,0.3

K,1,0,0,,

K,2,10,0,,

K,3,10,20,,

K,4,0,20,,

KPLOT

A,1,2,3,4

APLOT

TYPE,1

MAT,1

REAL,1

LESIZE,1,5

LESIZE,2,10

MSHKEY,1

AMESH,1

EPLOT

DL,1,,ALL,0

FINISH

/SOLU

ANTYPE,MODAL

MODOPT,LANB,10,0,500,

MXPAND,10

SOLVE

FINISH

/POST1

SET,LIST

SET,1,1

FINISH